MOV1                 C15

14D561K              470pF/250V

┌─┤├──┐              ┌─┤├──┐

│     │              │     │

AC (L) ─── F1 ──────┤     ├─── LF1 ──┬───┤     ├─── 1  AC(L)     +Vo  6 ───┬── L1 ──┬─────┬── +5V

300V/2A     │     │   UU9.8  │   │     │    │                 │    │  2.2uH │     │

│     │   30mH   │   │     │    │                 │    │       +++   ─┬─

├─┤├──┤         +++  ├─┤├──┤    │    AC-DC        │    │       │ │   TVS1

│ C14 │         │ │  │ C16 │    │                 │    │  C19  │ │  SMBJ

│0.1uF│         │ │  │470pF│┌── 3  +V(CAP)        │    │ 470uF │ │  7.0A

│/310V│         +++  │/250V││   │                 │    │  /25V +++   ─┴─

│     │   C17    │   │     │└── 4  -V(CAP) -Vo    5 ───┴───────┴───┬──┴─── GND

│     │  15uF/   │   │     │    │                 │                │

AC (N) ─────────────┤     ├──450V────┴───┤     ├─── 2  AC(N)          │               +++

│     │              │     │    │                 │               │ │ C20

└─────┘              └──┬──┘    └─────────────────┘               │ │ 470uF

│                                         +++  /10V

┤├ C18                                      │

1000pF/400V                                    │

│

═╧═

Earth

GND

F1          MOV1    C14         LF1                C15   C16

AC(L) ───────┤├─────┬─────┤├─────┤├────── pin1 ── pin3 ────┬────┤├────┘│

│                                      │          │

│     (both across L-N)                │         ═╧═ Earth

│                                      │          │

AC(N) ──────────────┴──────────────────── pin2 ── pin4 ────┬────┤├────┘

a      │    C18

│

┌─────────────────────────── │─────────────────┐

│                            │                 │

│  U1 (PBO-5F-5)             │                 │

│                            │                 │

│  pin 1  AC(L) ◄────────────┘                 │

│  pin 2  AC(N) ◄────────────┘                 │

│                                              │

│  pin 3  +V(CAP) ──┬──                        │

│                  C17  15uF/450V              │

│  pin 4  -V(CAP) ──┴──                        │

│                                              │

│  pin 6  +Vo ──┬── L1 ──┬───────┬──── +5V     │

│               │  2.2uH  │       │            │

│              C20       C19    TVS1           │

│          470uF/10V  470uF/25V  SMBJ7.0A      │

│               │        │       │             │

│  pin 5  -Vo ──┴────────┴───────┴──── GND     │

│                                              │

└──────────────────────────────────────────────┘

Signal flow summary

AC(L) → F1 → MOV1/C14 (across L-N) → LF1 → C15/C16/C18 (Y-caps to earth) → U1 pin 1

AC(N) → ──── MOV1/C14 (across L-N) → LF1 → C15/C16/C18 (Y-caps to earth) → U1 pin 2

U1 pin 3/4 → C17 (input bypass cap)

U1 pin 6 (+Vo) → L1 → C19/C20/TVS1 (all across +5V to GND) → +5V

U1 pin 5 (-Vo) → ──── C19/C20/TVS1 (all across +5V to GND) → GND

┌─────┬────────┬─────────┬──────────────────────────┬─────────────────────────────────────────┐

│ Ref │ Value  │ Voltage │           Type           │                 Purpose                 │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C14 │ 0.1uF  │ 310V    │ X2 safety cap            │ Differential EMC filtering (across L-N) │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C15 │ 470pF  │ 250V    │ Y1/Y2 safety cap         │ AC-side Y-cap to earth (L side)         │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C16 │ 470pF  │ 250V    │ Y1/Y2 safety cap         │ AC-side Y-cap to earth (N side)         │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C17 │ 15uF   │ 450V    │ Electrolytic             │ Input bypass on pins 3-4                │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C18 │ 1000pF │ 400V    │ Y1/Y2 safety cap         │ DC-side Y-cap to earth                  │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C19 │ 470uF  │ 25V     │ Solid-state electrolytic │ Bulk output filtering                   │

├─────┼────────┼─────────┼──────────────────────────┼─────────────────────────────────────────┤

│ C20 │ 470uF  │ 10V     │ Electrolytic             │ Secondary output filtering              │

└─────┴────────┴─────────┴──────────────────────────┴─────────────────────────────────────────┘

Other Components

┌──────┬─────────────────┬──────────────────────────────────────────┐

│ Ref  │  Value / Part   │                 Purpose                  │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ F1   │ 300V / 2A fuse  │ AC input protection                      │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ MOV1 │ 14D561K         │ Metal oxide varistor — surge suppression │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ LF1  │ UU9.8, 30mH min │ Common-mode choke — EMC filtering        │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ L1   │ 2.2uH           │ Output inductor                          │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ TVS1 │ SMBJ7.0A        │ TVS diode — output overvoltage clamp     │

├──────┼─────────────────┼──────────────────────────────────────────┤

│ U1   │ Bel PBO-5F-5    │ AC-DC power supply module                │

└──────┴─────────────────┴──────────────────────────────────────────┘

Make the Kemet symbol and footprint

The general approach — it's just 4 through-hole pads:

1. Open Footprint Editor

2. File > New Footprint

3. Name it CMC_KEMET_SSRH7H_18x14.5mm

4. Add 4 through-hole pads at the positions from the datasheet

- Hole size: likely ~0.8-1.0mm

- Pad size: ~1.6-1.8mm

5. Draw the component outline on F.SilkS (18.0 x 14.5mm rectangle)

6. Add ${REFERENCE} on F.Fab

7. Add courtyard on F.CrtYd with ~0.25mm margin around the body

8. Save

If you download the datasheet and either share the path or paste the pin dimensions, I can give you the exact pad coordinates.

※ recap: Designing the AC input and power supply circuit for the PelletPirate outdoor pellet grill controller board using a Bel PBO-5F-5. Currently need the KEMET SSRH7HS-M05324 common-mode choke datasheet to

build its KiCad footprint.